🐧 ISP (ALT Linux JEOS)

bash

hostnamectl set-hostname ISP

mkdir -p /etc/net/ifaces/ens19

mkdir -p /etc/net/ifaces/ens20

echo 'TYPE=eth' > /etc/net/ifaces/ens19/options

echo 'TYPE=eth' > /etc/net/ifaces/ens20/options

echo 'BOOTPROTO=static' >> /etc/net/ifaces/ens19/options

echo 'BOOTPROTO=static' >> /etc/net/ifaces/ens20/options

echo 172.16.1.1/28 > /etc/net/ifaces/ens19/ipv4address

echo 172.16.2.1/28 > /etc/net/ifaces/ens20/ipv4address

systemctl restart network

apt-get update && apt-get install tzdata nftables -y

timedatectl set-timezone Europe/Moscow

cat <<'EOT' > /etc/nftables.nft

#!/usr/sbin/nft -f

flush ruleset

table ip nat {

chain postrouting {

type nat hook postrouting priority srcnat; policy accept;

oifname "ens18" masquerade

}

}

EOT

systemctl enable --now nftables

echo "net.ipv4.ip_forward = 1" >> /etc/net/sysctl.conf

sysctl -p

cat /proc/sys/net/ipv4/ip_forward

🍃 HQ-RTR (EcoRouter) – полная конфигурация

bash

en

conf t

hostname hq-rtr.au-team.irpo

ip domain-name au-team.irpo

int ISP

ip address 172.16.1.2/28

ip nat outside

exit

port te0

service-instance ISP

encapsulation untagged

connect ip interface ISP

exit

exit

int 100

ip address 192.168.2.1/27

ip nat inside

exit

int 200

ip address 192.168.2.33/27

ip nat inside

exit

int 999

ip address 192.168.2.65/29

ip nat inside

exit

port te1

service-instance 100

encapsulation dot1q 100

rewrite pop 1

connect ip interface 100

exit

service-instance 200

encapsulation dot1q 200

rewrite pop 1

connect ip interface 200

exit

service-instance 999

encapsulation dot1q 999

rewrite pop 1

connect ip interface 999

exit

exit

ip route 0.0.0.0/0 172.16.1.1

ntp timezone UTC+3

username net_admin

password P@ssw0rd

role admin

ip pool dhcp 1

range 192.168.2.34-192.168.2.62

exit

dhcp-server 1

pool dhcp 64

mask 255.255.255.224

gateway 192.168.2.33

dns 192.168.2.2

domain-name au-team.irpo

exit

interface 200

dhcp-server 1

exit

ip nat pool nat 192.168.2.1-192.168.2.30,192.168.2.33-192.168.2.62,192.168.2.65-192.168.2.70

ip nat source dynamic inside-to-outside pool nat overload interface ISP

interface tunnel.1

ip add 10.10.10.1/30

ip tunnel 172.16.1.2 172.16.2.2 mode gre

ip ospf authentication

ip ospf authentication-key P@ssw0rd

exit

router ospf 1

passive-interface default

no passive-interface tunnel.1

network 10.10.10.0/30 area 0

network 192.168.2.0/27 area 0

network 192.168.2.32/27 area 0

network 192.168.2.64/29 area 0

area 0 authentication

exit

wr mem

🍃 BR-RTR (EcoRouter) – полная конфигурация

bash

en

conf t

hostname br-rtr.au-team.irpo

ip domain-name au-team.irpo

int ISP

ip address 172.16.2.2/28

ip nat outside

exit

port te0

service-instance ISP

encapsulation untagged

connect ip interface ISP

exit

exit

int 100

ip address 192.168.3.1/28

ip nat inside

exit

port te1

service-instance 100

encapsulation untagged

connect ip interface 100

exit

exit

ip route 0.0.0.0/0 172.16.2.1

ntp timezone UTC+3

username net_admin

password P@ssw0rd

role admin

ip nat pool nat 192.168.3.1-192.168.3.14

ip nat source dynamic inside-to-outside pool nat overload interface ISP

interface tunnel.1

ip add 10.10.10.2/30

ip tunnel 172.16.2.2 172.16.1.2 mode gre

ip ospf authentication

ip ospf authentication-key P@ssw0rd

exit

router ospf 1

passive-interface default

no passive-interface tunnel.1

network 10.10.10.0/30 area 0

network 192.168.3.0/28 area 0

area 0 authentication

exit

wr mem

🐧 HQ-SRV (ALT Linux)

bash

hostnamectl set-hostname hq-srv.au-team.irpo

cat > /etc/net/ifaces/ens18/options << EOF

BOOTPROTO=static

TYPE=eth

CONFIG_WIRELESS=no

SYSTEMD_BOOTPROTO=static

CONFIG_IPV4=yes

DISABLED=no

NM_CONTROLLED=no

SYSTEMD_CONTROLLED=no

EOF

echo 192.168.2.2/27 > /etc/net/ifaces/ens18/ipv4address

echo default via 192.168.2.1 > /etc/net/ifaces/ens18/ipv4route

cat > /etc/net/ifaces/ens18/resolv.conf << EOF

search au-team.irpo

nameserver 77.88.8.8

EOF

systemctl restart network

timedatectl set-timezone Europe/Moscow

useradd -s /bin/bash -u 2026 sshuser

echo "sshuser:P@ssw0rd" | chpasswd

gpasswd -a sshuser wheel

echo 'sshuser ALL = (root) NOPASSWD: ALL' >> /etc/sudoers

sed -i 's/^#Port 22/Port 2026/' /etc/openssh/sshd_config

sed -i 's/^#MaxAuthTries 6/MaxAuthTries 2/' /etc/openssh/sshd_config

echo "Banner /etc/openssh/banner" >> /etc/openssh/sshd_config

echo "AllowUsers sshuser" >> /etc/openssh/sshd_config

echo "Authorized access only" > /etc/openssh/banner

systemctl restart sshd

apt-get update && apt-get install dnsmasq -y

echo 'OPTIONS=""' > /etc/sysconfig/dnsmasq

cat > /etc/dnsmasq.conf << EOF

no-resolv

no-poll

no-hosts

server=77.88.8.7

server=195.208.4.1

server=8.8.8.8

cache-size=1000

all-servers

no-negcache

host-record=hq-rtr.au-team.irpo,192.168.2.1

ptr-record=1.2.168.192.in-addr.arpa,hq-rtr.au-team.irpo

host-record=hq-srv.au-team.irpo,192.168.2.2

ptr-record=2.2.168.192.in-addr.arpa,hq-srv.au-team.irpo

host-record=hq-cli.au-team.irpo,192.168.2.34

ptr-record=34.2.168.192.in-addr.arpa,hq-cli.au-team.irpo

address=/br-rtr.au-team.irpo/192.168.3.1

address=/br-srv.au-team.irpo/192.168.3.2

address=/docker.au-team.irpo/172.16.1.1

address=/web.au-team.irpo/172.16.2.1

EOF

systemctl enable --now dnsmasq

🐧 BR-SRV (ALT Linux)

bash

hostnamectl set-hostname br-srv.au-team.irpo

cat > /etc/net/ifaces/ens18/options << EOF

BOOTPROTO=static

TYPE=eth

CONFIG_WIRELESS=no

SYSTEMD_BOOTPROTO=static

CONFIG_IPV4=yes

DISABLED=no

NM_CONTROLLED=no

SYSTEMD_CONTROLLED=no

EOF

echo 192.168.3.2/28 > /etc/net/ifaces/ens18/ipv4address

echo default via 192.168.3.1 > /etc/net/ifaces/ens18/ipv4route

cat > /etc/net/ifaces/ens18/resolv.conf << EOF

search au-team.irpo

nameserver 192.168.2.2

EOF

systemctl restart network

timedatectl set-timezone Europe/Moscow

useradd -s /bin/bash -u 2026 sshuser

echo "sshuser:P@ssw0rd" | chpasswd

gpasswd -a sshuser wheel

echo 'sshuser ALL = (root) NOPASSWD: ALL' >> /etc/sudoers

sed -i 's/^#Port 22/Port 2026/' /etc/openssh/sshd_config

sed -i 's/^#MaxAuthTries 6/MaxAuthTries 2/' /etc/openssh/sshd_config

echo "Banner /etc/openssh/banner" >> /etc/openssh/sshd_config

echo "AllowUsers sshuser" >> /etc/openssh/sshd_config

echo "Authorized access only" > /etc/openssh/banner

systemctl restart sshd

cat > /etc/apt/apt.conf << EOF

Acquire::http::Proxy "http://10.0.21.52:3128/";

Acquire::https::Proxy "http://10.0.21.52:3128/";

EOF

🐧 HQ-CLI (ALT Linux с графикой – DHCP)

bash

hostnamectl set-hostname hq-cli.au-team.irpo

cat > /etc/net/ifaces/ens18/options << EOF

BOOTPROTO=dhcp

TYPE=eth

CONFIG_WIRELESS=no

SYSTEMD_BOOTPROTO=dhcp

CONFIG_IPV4=yes

DISABLED=no

NM_CONTROLLED=no

SYSTEMD_CONTROLLED=no

EOF

systemctl restart network

timedatectl set-timezone Europe/Moscow